附件1：试剂参数要求

| 试剂名称 | 规格 | 参数需求 |
| --- | --- | --- |
| 醋酸铵 | 500g | 色谱级 纯度（氢氧化钠滴定法）≥99% 280 nm紫外吸收 0-0.02 ph 6.5-7.5 260nm紫外吸光度 0-0.04 250 nm紫外吸收 0-0.05 270nm紫外吸收 0-0.03 500 nm紫外吸收 0-0.02 |
| 醋酸钾 | 500g | 分析纯AR 纯度（高氯酸滴定法）≥99% |
| 甲酸铵 | 500g | 超纯 纯度（中和滴定法）≥99% 铜（Cu） 0-50ppm 镍（Ni） 0-50ppm 钾（K）0-200ppm 铁（Fe）0-50ppm 铅（Pb）0-50ppm 钠（Na）0-100ppm 锌（Zn）0-50ppm 钴（Co）0-50ppm 硫酸盐（SO42-）0-100ppm 水（卡尔-费舍法）0-3% 钙（Ca）0-50ppm 氯离子（Cl-）0-50ppm 镉（Cd）0-50ppm pH(25℃，50mg／mL水溶液)6-7 |
| 碳酸氢铵 | 500g | 超纯生物试剂 含量≥99.5% 点火残留物 0-0.05% 铝（AI）0-5( ppm) 钡（Ba）0-5( ppm) 铋（Bi）0-5( ppm) 钙（Ca）0-10( ppm) 镉（Cd）0-5( ppm) 钴（Co） 0-5( ppm) 铬（ICP）0-5( ppm) 铜（Cu）0-5( ppm) 铁（Fe）0-5( ppm) 钾(K) 0-50( ppm) 锂（Li） 0-5( ppm) 镁（Mg）0-500( ppm) 锰（Mn）0-5( ppm) 钼（Mo）0-5( ppm) 钠（Na）0-50( ppm) 镍（Ni）0-5( ppm) 铅（Pb）0-5( ppm) 锶（Sr）0-5( ppm) 锌（Zn）0-5( ppm) 砷痕迹（MHS-AAS） 0-0.1( ppm) 总硫，以硫酸盐计（ICP法） 0-50( ppm) 氯离子（Cl-）0-50( ppm) pH（25℃，1 M H2O中）7-8.5 |
| 碳酸钾 | 500g | 分析纯AR 含量≥99.0% 灼烧失量≤1.0% 氯化物(Cl)≤0.003% 硫化合物(以SO4计)≤0.003% 总氮量(N)≤0.001% 磷酸盐及硅酸盐(以SiO2计)≤0.005% 钠(Na)≤0.05% 镁(Mg)≤0.002% 钙(Ca)≤0.002% 铁(Fe)≤0.0005% 铜(Cu)≤0.0005% 铅(Pb)≤0.0005% 水不溶物≤0.005% |
| 碳酸氢钾 | 500g | 纯度（用HCL滴定法测定）（干基）≥99.7% 水中的不溶物0-0.01（%） 重金属（如Pb）0-5（ppm） 磷酸盐 (PO4) 0-5( ppm) 铁（Fe）0-5（ppm） 钠（Na）0-0.03（%） 镁（Mg）0-0.001（%） 氯化物0-0.001（%） 钙（Ca）0-0.002（%） 氨（NH4）0-5（ppm） 硫酸盐（SO42-)0-0.003（%） 痕量金属分析0-100（ppm） |
| 二水合磷酸二氢钠 | 500g | 二水合磷酸二氢钠 分析纯AR 含量≥99.0% PH(50g／L，25℃) 4.2~4.6 水不溶物≤0.01% 氯化物(Cl)≤0.005% 硫酸盐(SO4)≤0.005% 硝酸盐(NO3)≤0.001% 铁(Fe)≤0.001% 重金属(以Pb计)≤0.001% 氨沉淀物≤0.01% 钾(K)≤0.02% 铵(NH4)≤0.002% 砷(As)≤0.0002% |
| 磷酸氢二钠 | 500g | 分析纯AR 含量(Na2HPO4)≥99.0% 总氮量(N）≤0.002% 氯化物(Cl)≤0.002% 硫酸盐(SO4)≤0.005% 干燥失量≤0.5% 水不溶物≤0.01% 重金属(以Pb计)≤0.001% 铁(Fe)≤0.002% 砷(As)≤0.0001% pH(50g/L溶液，25℃)8.7～9.3 |
| 磷酸 | 100ml | 纯度（基于痕量金属分析） ≥99.99% 金属杂质总量 0-200ppm |
| K222（4,7,13,16,21,24-六氧-1,10-二氮双环[8.8.8]二十六烷）（CAS号23978-09-8） | 1g | 纯度≥99% |
| - 二甲基氨基乙醇（DMEA） （CAS号108-01-0） | 100ml | 纯度≥99.5% |
| 二甲基亚砜（DMSO） （CAS号 67-68-5） | 100ml | 纯度≥99.7% 水份≤0.1% 蒸发残留≤0.01% 可滴定酸≤0.001 meq/g |
| 超干二甲基甲酰胺（DMF） （CAS号68-12-2） | 100ml | 纯度≥99.75% 水分≤0.005% 蒸发残留≤0.0005% |
| 碘单质 | 250g | 分析纯AR 含量≥99.8% 蒸发残渣≤0.005% 氯及溴(以Cl计)≤0.005% |
| 乙醇 | 4L | HPLC级 400nm紫外吸光度≤0.01 270nm紫外吸光度≤0.01 240nm紫外吸光度≤0.05 230nm紫外吸光度≤ 0.15 220nm紫外吸光度≤0.25 210nm紫外吸光度≤0.40 水份≤0.02 % 纯度(GC)≥99.5% 蒸发残留物≤0.001 % |
| 乙腈 | 4L | HPLC级 400nm紫外吸光度≤0.0044 250nm紫外吸光度 ≤0.0044 235nm紫外吸光度 ≤0.0044 230nm紫外吸光度 ≤0.0044 200nm紫外吸光度 ≤0.032 195nm紫外吸光度 ≤0.12 纯度(GC)≥99.90% 蒸发残留物≤0.0005 % 水份≤0.02 % 254nm荧光（奎宁）≤1.0 ppb 365nm荧光（奎宁）≤0.5ppb HPLC 210 nm吸光度≤3mAU HPLC 254 nm吸光度≤0.5mAU 210nm 基线漂移≤15mAU 碱度（NH3）≤0.0002% 游离酸 (醋酸)≤0.001 % |
| 甲醇 | 1L | HPLC级 400nm紫外吸收≤0.01 280nm紫外吸收≤0.01 260nm紫外吸收≤0.04 240nm紫外吸收≤0.10 235nm紫外吸收≤ 0.10 230nm紫外吸收≤ 0.20 220nm紫外吸收≤ 0.30 210nm 紫外吸收≤0.60 205nm紫外吸收≤1.00 纯度(GC) ≥ 99.90 % 蒸发残留物 ≤ 0.0005 % 水份≤ 0.03 % 254nm荧光≤1 ppb 365nm荧光≤1 ppb |
| 丙酮 | 1L | HPLC级 400nm紫外吸收≤0.005 375nm 紫外吸收≤0.005 350nm紫外吸收≤0.01 340nm紫外吸收≤0.06 330nm紫外吸收≤1.0 纯度 (GC) > 99.90 % 蒸发残留物 ≤ 0.0003 % 水份≤0.5 % 365nm荧光（奎宁）≤1.0 ppb |
| 异丙醇 | 1kg | 纯度(GC)≥99.7% 无单一杂质≥1000ppm 所有杂质总和≤5000 ppm 甲醇含量≤200 ppm 水(卡尔费休法)≤0.2% 蒸发残留物≤10 ppm 砷(As)≤3ppm 镉(Cd)≤1ppm 汞(Hg)≤1ppm 铅(Pb)≤1ppm |
| 盐酸HCl | 250ml | 盐酸含量：36-38% 铝＜50ppt 锑＜50ppt 钡＜50ppt 铍＜50ppt 铋＜50ppt 镉＜50ppt 钙＜50ppt 铬＜50ppt 钴＜50ppt 铜＜50ppt 镓＜50ppt 锗＜50ppt 金＜50ppt 铟＜50ppt 铁＜50ppt 铅＜50ppt 锂＜50ppt 镁＜50ppt 锰＜50ppt 钼＜50ppt 镍＜50ppt 铌＜50ppt 钾＜50ppt 硒＜50ppt 银＜50ppt 钠＜50ppt 锶＜50ppt 钽＜50ppt 铊＜50ppt 锡＜50ppt 钛)＜50ppt 钒＜50ppt 锌＜50ppt 锆＜50ppt |
| 超纯水 | 1000ml | 银≤50ppt 铝≤200ppt 砷≤50ppt 金≤50ppt 硼≤500ppt 钡≤50ppt 铍≤50ppt Bi(Bismuth)≤50ppt Ca(Calcium)≤500ppt Cd (Cadmium)≤50ppt Ce(Cerium)≤20.0ppt Co (Cobalt)≤50ppt Cr(Chromium)≤50ppt Cu(Copper)≤100ppt Dy(Dysprosium)≤20.0ppt Er(Erbium)≤20.0ppt Eu(Europium)≤20.0ppt Fe(Iron)≤300ppt Ga(Gallium)≤50.0ppt Gd (Gadolinium)≤20.0ppt Ge(Germanium)≤50ppt Hf(Hafnium)≤10.0ppt Ho (Holmium)≤20.0ppt In(Indium)≤50.0ppt K(Potassium)≤300ppt La(Lanthanum)≤20.0ppt Li(Lithium)≤50.0ppt Lu(Lutetium)≤20.0ppt Mg(Magnesium)≤100ppt Mn(Manganese)≤50ppt Mo(Molybdenum)≤50ppt Na(Sodium)≤500ppt Nb (Niobium)≤20.0ppt Nd(Neodymium)≤20.0ppt Ni(Nickel)≤50ppt Pb (Lead)≤50ppt Pd (Palladium)≤50ppt Pr (Praseodym)≤20.0ppt Pt(Platinum)≤50ppt Rb (Rubidium)≤20.0ppt Sb (Antimony)≤50ppt Sc(Scandium)≤20ppt Si(Silicon)≤1000ppt Sm (Samarium)≤20.0ppt Sn(Tin)≤50ppt Sr(Strontium)≤50.0ppt Ta(Tantalum)≤50.0ppt Tb(Terbium)≤20.0ppt Te(Tellur)≤50ppt Th (Thorium)≤10.00ppt Ti(Titanium)≤50ppt TI(Thallium)≤50.0ppt Tm(Thulium)≤20.0ppt U(Uranium)≤10.00ppt V(Vanadium)≤50ppt W(Wolfram)≤100ppt Y(Yttrium)≤10.0ppt Yb (Ytterbium)≤20.0ppt Zn(Zinc)≤500ppt Zr(Zirconium)≤50.0ppt |
| 草酸二水合物 | 100g | 纯度≥99.5%， Chloride(Cl)≤5000ppb Phosphate (PO4)≤500ppb Sulfate(SO4)≤2000ppb Ag(Silver)≤10ppb Al(Aluminium)≤20ppb As(Arsenic)≤1.0ppb Au(Gold)≤1.0ppb Ba(Barium)≤100ppb Be(Beryllium)≤1.0ppb Bi(Bismuth)≤1.0ppb Ca(Calcium)≤100ppb Cd(Cadmium)≤50ppb Co (Cobalt)≤5ppb Cr(Chromium)≤10ppb Cu(Copper)≤5ppb Fe(Iron)≤50ppb Ga(Gallium)≤1.0ppb Ge(Germanium)≤1.0ppb In(Indium)≤1.0ppb K(Potassium)≤200ppb Li(Lithium)≤5ppb Mg(Magnesium)≤20ppb Mn(Manganese)≤5ppb Mo(Molybdenum)≤5ppb Na(Sodium)≤100ppb Ni(Nickel)≤10ppb Pb (Lead)≤10ppb Pt(Platinum)≤1.0ppb Sb(Antimony)≤1.0ppb Sn (Tin)≤5ppb Sr(Strontium)≤100ppb Ti(Titanium)≤5ppb TI(Thallium)≤1.0ppb U (Uranium)≤1.0ppb V(Vanadium)≤10ppb Zn(Zinc)≤20ppb |
| 过氧化氢H2O2 | 500ml | 含量≥30.0% 蒸发残渣≤0.0025% 酸度mmol/100g≤0.06 氯化物（CL）≤0.00005% 硫酸盐（SO4）≤0.0002% 总氮量（N）≤0.0004% 磷酸盐（PO4）≤0.0002% 砷（As）≤0.00005% 铁（Fe）≤0.00001% 镍（Ni）≤0.000002% 铜（Cu）≤0.000002% 铅（Pb）≤0.000002% |
| 氯化钠 | 250g | 纯度（金属法）≥99.99% 含量测定（银量法）≥99.5% Bromide(Br)≤10ppm lodide(I)≤10ppm Phosphate(PO₄)≤5ppm Sulfate(SO4)≤10ppm Total nitrogen(N)≤10ppm Hexacyanoferrate(Fe(CN)₆)≤1.0ppm Al(Aluminium)≤0.010ppm Ba(Barium)≤2.0ppm Ca(Calcium)≤0.10ppm Cd(Cadmium)≤0.005ppm Ce(Cerium)≤0.005ppm Co (Cobalt)≤0.005ppm Cr(Chromium)≤0.010ppm Cs(Cesium)≤5ppm Cu(Copper)≤0.005ppm Eu(Europium)≤0.005ppm Fe(Iron)≤0.050ppm K(Potassium)≤5.0ppm La(Lanthanum)≤0.005ppm Li(Lithium)≤0.5ppm Mg(Magnesium)≤0.05ppm Mn(Manganese)≤0.010ppm Ni(Nickel)≤0.010ppm Pb (Lead)≤0.005ppm Rb (Rubidium)≤0.5ppm Sc(Scandium)≤0.005ppm Sm(Samarium)≤0.005ppm Sr(Strontium)≤0.10ppm TI(Thallium)≤0.005ppm Y(Yttrium)≤0.005ppm Yb(Ytterbium)≤0.005ppm Zn(Zinc)≤0.020ppm |